3.1 Show Debugging Features

Above shows how the code stops when we try to construct xx from only 4 samples of the vector tt

3.2

3.3.1

Period is .3343 seconds and is represents the fundamental frequency of 3 Hz, the GCD of all the frequencies used to construct this signal.

3.3.2

nStart = round(fs * tStart(kk))+1; %-- add one to avoid zero index

nStop = nStart+Lnew-1; %<========= Add code

3.4

testingBeat.values = real( testingBeat.X1*exp(j*2*pi*testingBeat.f1 * testingBeat.times ) + testingBeat.X2*exp(j*2*pi*testingBeat.f2 * testingBeat.times ) );

where the values from testingBeat struct is the values of the generated beat signal.

function sigBeatSum = sum2BeatStruct( sigBeatIn ) %

%--- Assume the five basic fields are present, plus the starting and ending times

%--- Add the four fields for the parameters in Equation (4)

%

% sigBeatSum.f1, sigBeatSum.f2, sigBeatSum.X1, sigBeatSum.X2

sigBeatSum.f1 = sigBeatIn.fc-sigBeatIn.fDelt;

sigBeatSum.f2 = sigBeatIn.fc+sigBeatIn.fDelt;

% Amplitude --> See Eq. (4)

A1 = sigBeatIn.Amp / 2;

A2 = sigBeatIn.Amp / 2;

% Phase --> See Eq. (4)

phi1 = sigBeatIn.phic - sigBeatIn.phiDelt;

phi2 = sigBeatIn.phic + sigBeatIn.phiDelt;

% Compute complex amplitude

sigBeatSum.X1 = A1 * exp(j * phi1);

sigBeatSum.X2 = A2 * exp(j * phi2);

end

3.5

Code for all

%% 3.2

mySig.freq = 3; %-- (in hertz)

mySig.complexAmp = 4*exp(j*pi/6);

dur = 3;

start = -1;

dt = 1/(32*mySig.freq);

mySigWithVals = makeCosVals( mySig, dur, start, dt );

dbstop if error

%- Plot the values in sigWithVals

plot( mySigWithVals.times, mySigWithVals.values)   %<--- complete the plot statement

xlabel('t [s]')

ylabel('x(t)')

function sigOut = makeCosVals(sigIn, dur, tstart, dt ) %

freq = sigIn.freq;

X = sigIn.complexAmp;

%

%...(Fill in several lines of code)...

%

A = abs(sigIn.complexAmp);

tt = tstart: dt : dur + tstart%-- Create the vector of times

xx = A*cos(2 * pi * sigIn.freq * tt + angle(sigIn.complexAmp));     %-- Vectorize the cosine evaluation

sigOut.times = tt;     %-- Put vector of times into the output structure

sigOut.values = xx;    %-- Put values into the output structure

end

%% 3.3.1

ss(1).freq = 21; ss(1).complexAmp = exp(j*pi/4);

ss(2).freq = 15; ss(2).complexAmp = 2i;

ss(3).freq = 9; ss(3).complexAmp = -4;

%

dur = 1;

tstart = -0.5;

dt = 1/(21*32); %-- use the highest frequency to define delta_t

%

ssOut = addCosVals( ss, dur, tstart, dt );

%

plot( ssOut.times, ssOut.values) %

function sigOut = addCosVals( cosIn, dur, tstart, dt )

%ADDCOSVALS Synthesize a signal from sum of sinusoids

%(do not assume all the frequencies are the same)

%

% usage: sigOut = addCosVals( cosIn, dur, tstart, dt )

%

% cosIn = vector of structures; each one has the following fields:

%   cosIn.freq = frequency (in Hz), usually none should be negative

%   cosIn.complexAmp = COMPLEX amplitude of the cosine

%

% dur = total time duration of all the cosines

% start = starting time of all the cosines

% dt = time increment for the time vector

%

% The output structure has only signal values because it is not necessarily a sinusoid

%   sigOut.values = vector of signal values at t = sigOut.times

%   sigOut.times = vector of times, for the time axis

%

% The sigOut.times vector should be generated with a small time increment that

%   creates 32 samples for the shortest period, i.e., use the period

%   corresponding to the highest frequency cosine in the input array of structures.

% <--- Write your code here --->

tt = tstart:dt:dur;

xx = 0*tt;

for k = 1:length(cosIn)

xNew = abs(cosIn(k).complexAmp) * cos(2*pi*cosIn(k).freq*tt + angle(cosIn(k).complexAmp));

xx(1:length(tt)) = xx(1:length(tt))+xNew;

end

sigOut.times = tt;

sigOut.values = xx;

end

%% 3.3.2

% Modify the following code from Prelab 2.6

amps = [1, 1] ;

freqs = [1200, 750] ;

phases = [0, 0] ;

fs = 4000;

tStart = [.6, .2];

durs = [.5, 1.6];

maxTime = max(tStart+durs) + 0.1; %-- Add time to show signal ending

durLengthEstimate = ceil(maxTime*fs);

tt = (0:durLengthEstimate)*(1/fs); %-- be conservative (add one)

xx = 0*tt; %--make a vector of zeros to hold the total signal

for kk = 1:length(amps)

nStart = round(fs * tStart(kk))+1; %-- add one to avoid zero index

xNew = shortSinus(amps(kk), freqs(kk), phases(kk), fs, durs(kk));

Lnew = length(xNew);

nStop = nStart+Lnew-1; %<========= Add code

xx(nStart:nStop) = xx(nStart:nStop) + xNew;

end

tt = (1/fs)*(0:length(xx)-1);

figure

spectrogram(xx,256,[ ],[ ],fs,'yaxis'); colorbar

function xs = shortSinus(amp, freq, pha, fs, dur)

% amp = amplitude

% freq = frequency in cycle per second

% pha = phase, time offset for the first peak

% fs = number of sample values per second

% dur = duration in sec

%

tt = 0 : 1/fs : dur; % time indices for all the values

xs = amp * cos( freq*2*pi*tt + pha );

end

%% 3.4

%% Template of sigBeat Struct

% sigBeat.Amp = 10;     %-- B in Equation (3)

% sigBeat.fc = 480;     %-- center frequency in Eq. (3)

% sigBeat.phic = 0;     %-- phase of 2nd sinusoid in Eq. (3)

% sigBeat.fDelt = 20;   %-- modulating frequency in Eq. (3)

% sigBeat.phiDelt = -2*pi/3;    %-- phase of 1st sinusoid Eq. (3)

% sigBeat.t1 = 1.1;     %-- starting time

% sigBeat.t2 = 5.2;     %-- ending time %

%

%----- extra fields for the parameters in Equation (4)

%

% sigBeat.f1        %-- frequencies in Equation (4)

% sigBeat.f2        %--

% sigBeat.X1        %-- complex amps for sinusoids in Equation (4)

% sigBeat.X2        %-- derived from A’s and phi’s

%

% sigBeat.values    %-- vector of signal values sigBeat.times

% sigBeat.times     %-- vector of corresponding times

%% 3.4(a)

% Complete the sum2BeatStruct function at the end

%% 3.4(b)

% Create a beat signal with two frequency components:

% one at 720 Hz and one at 750 Hz

fs = 2000;

sigBeat.Amp = 10; %-- B in Equation (3)

sigBeat.fc = 735; %-- center frequency in Eq. (3)

sigBeat.phic = 0; %-- phase of 2nd sinusoid in Eq. (3)

sigBeat.fDelt = 15; %-- modulating frequency in Eq. (3)

sigBeat.phiDelt = 0; %-- phase of 1st sinusoid Eq. (3)

sigBeat.t1 = 0; %-- starting time

sigBeat.t2 = 4; %-- ending time %

testingBeat = sum2BeatStruct( sigBeat );

testingBeat.times = sigBeat.t1:1/fs:sigBeat.t2;

testingBeat.values = real( testingBeat.X1*exp(j*2*pi*testingBeat.f1 * testingBeat.times + angle(testingBeat.X1)) ...

+ testingBeat.X2*exp(j*2*pi*testingBeat.f2 * testingBeat.times + angle(testingBeat.X2)) );

figure

spectrogram(testingBeat.values,1024,[ ],[ ],fs,'yaxis'); colorbar

soundsc(testingBeat.values, fs)

%% 3.4.1(a)

fs = 8000;

sigBeat.Amp = 50; %-- B in Equation (3)

sigBeat.fc = 800; %-- center frequency in Eq. (3)

sigBeat.phic = 0; %-- phase of 2nd sinusoid in Eq. (3)

sigBeat.fDelt = 30; %-- modulating frequency in Eq. (3)

sigBeat.phiDelt = pi/4; %-- phase of 1st sinusoid Eq.~(3)

sigBeat.t1 = 0; %-- starting time

sigBeat.t2 = 4.04; %-- ending time %

testingBeat = sum2BeatStruct( sigBeat );

testingBeat.times = sigBeat.t1:1/fs:sigBeat.t2;

testingBeat.values = real( testingBeat.X1*exp(j*2*pi*testingBeat.f1 * testingBeat.times ) ...

+ testingBeat.X2*exp(j*2*pi*testingBeat.f2 * testingBeat.times ) );

figure

plot( testingBeat.times(1:500), testingBeat.values(1:500) )

%% 3.4.1(c)

plotspec(testingBeat.values+j*1e-12,fs,512); grid on, shg

%%

function sigBeatSum = sum2BeatStruct( sigBeatIn ) %

%--- Assume the five basic fields are present, plus the starting and ending times

%--- Add the four fields for the parameters in Equation (4)

%

% sigBeatSum.f1, sigBeatSum.f2, sigBeatSum.X1, sigBeatSum.X2

sigBeatSum.f1 = sigBeatIn.fc-sigBeatIn.fDelt;

sigBeatSum.f2 = sigBeatIn.fc+sigBeatIn.fDelt;

% Amplitude --> See Eq. (4)

A1 = sigBeatIn.Amp / 2;

A2 = sigBeatIn.Amp / 2;

% Phase --> See Eq. (4)

phi1 = sigBeatIn.phic - sigBeatIn.phiDelt;

phi2 = sigBeatIn.phic + sigBeatIn.phiDelt;

% Compute complex amplitude

sigBeatSum.X1 = A1 * exp(j * phi1);

sigBeatSum.X2 = A2 * exp(j * phi2);

end

%% 3.5

myLFMsig.f1 = 200;

myLFMsig.t1 = 0;

myLFMsig.t2 = 1.5;

myLFMsig.slope = 1800;

myLFMsig.complexAmp = 10*exp(j*0.3*pi);

dt = 1/8000; % 8000 samples per sec is the sample rate

outLFMsig = makeLFMvals(myLFMsig,dt);

%- Plot the values in outLFMsig

plot(outLFMsig.times, outLFMsig.values)

%- Make a spectrogram for outLFMsig to see the linear frequency change

spectrogram(outLFMsig.values, 512,[ ],[ ],1/dt,'centered','yaxis')

function sigOut = makeLFMvals( sigLFM, dt )

% MAKELFMVALS       generate a linear-FM chirp signal

%

% usage: sigOut = makeLFMvals( sigLFM, dt )

% sigLFM.f1 = starting frequency (in Hz) at t = sigLFM.t1

% sigLFM.t1 = starting time (in secs)

% sigLFM.t2 = ending time

% sigLFM.slope = slope of the linear-FM (in Hz per sec)

% sigLFM.complexAmp = defines the amplitude and phase of the FM signal

% dt = time increment for the time vector, typically 1/fs (sampling frequency)

%

% sigOut.values = (vector of) samples of the chirp signal

% sigOut.times = vector of time instants from t=t1 to t=t2

%

if( nargin < 2 ) %-- Allow optional input argument for dt

dt = 1/8000; %-- 8000 samples/sec

end

%--------NOTE: use the slope to determine mu needed in psi(t)

%-------- use f1, t1 and the slope to determine f0 needed in psi(t)

tt = sigLFM.t1:dt:sigLFM.t2;

mu = .5*(sigLFM.slope);

f0 = sigLFM.f1;

psi = 2*pi*( f0*tt + mu*tt.*tt) + angle(sigLFM.complexAmp);

xx = real( abs(sigLFM.complexAmp) * exp(j*psi) );

sigOut.times = tt;

sigOut.values = xx;

end