Rudra Goel

Lab 06 Report

ECE 2031 L10

06 October 2024

Figure 1. Worksheet diagram of state A1CCW_B3CCW where Train A is on track 1 going counterclockwise, and Train B is on track 3 going counterclockwise. At this state, the switches do not need to be asserted to any logic level. When sensor 1 (S1) is asserted, state transitions to Aarrived_B3CCW; when sensor 2 (S2) is asserted, state moves to A1CCW_B2CCW; when both sensors are asserted (S1∙S2), state transitions to Aarrived_B2CCW.

Figure 2. Worksheet diagram of state Aarrived_B3CCW where Train A is stopped at sensor 1, and Train B is on track 3 going counterclockwise. At this state, the switches do not need to be asserted to any logic level. When sensor 2 (S2) is asserted, state moves to Aarrived_B2CCW.

Figure 3. Worksheet diagram of state A1CCW_B2CCW where Train A is on track 1 going counterclockwise, and Train B is on track 2 going counterclockwise. Switch 1 is asserted logic high to connect track 3 to track 2. Switch two is asserted logic low to keep track 2 connected with itself. When sensor 1 (S1) is asserted, state transitions to Aarrived_B2CCW; when sensor 4 (S4) is asserted, state moves to A1CCW_B2CW; when both sensors are asserted (S1∙S4), state transitions to Aarrived_B2CW.

Figure 4. Worksheet diagram of state Aarrived_B2CCW where Train A is stationary at sensor 1, and Train B is on track 2 going counterclockwise. Switch 1 is asserted logic high to connect track 3 to track 2. Switch 2 is asserted logic low to keep track 2 connected. When sensor 4 (S4) is asserted, state moves to A1CCW_B2CW.

Figure 5. Worksheet diagram of state Aarrived_B2CW where Train A is stationary at sensor 1, and Train B is on track 2 going counterclockwise. Switch 1 is asserted logic low to connect track 1 to track 2. Switch 2 is asserted logic low to keep track 2 connected with itself. There is no next state as this state is certain to crash the trains at sensor 1.

Figure 6. Worksheet diagram of state A1CCW_B2CW where Train A is on track 1 going counterclockwise, and Train B is on track 2 going clockwise. At this state, switch 1 is asserted to logic high to connect track 3 to track 2. Switch 2 is asserted logic low to keep track 2 connected with itself. When sensor 1 (S1) is asserted, state transitions to Aarrived_B2CW; when sensor 2 (S4) is asserted, state moves to A1CCW_B2CCW.

Table 1

Table Of Outputs For Train Controller State Machine

Figure 7. UML state diagram for Train Controller state machine meant to crash both trains at sensor 1. Initial state marked by black dot is A1CCW_B3CCW. Crash state is Aarrived_B2CW. Train B bounces back-and-forth on track 2 until Train A arrives at sensor 1 where it then travels clockwise on track 2 onto track 1 to crash at sensor 1.

Appendix A

VHDL Code Implementing Train Controller Finite State Machine

-- State machine to control trains, adapted from class template

-- Author: Rudra Goel

-- Date 10/3/2024

LIBRARY IEEE;

USE  IEEE.STD_LOGIC_1164.all;

USE  IEEE.STD_LOGIC_ARITH.all;

USE  IEEE.STD_LOGIC_UNSIGNED.all;

ENTITY TrainController IS

PORT(

reset, clock, sensor1, sensor2      : IN std_logic;

sensor3, sensor4, sensor5, sensor6  : IN std_logic;

switch1, switch2, switch3, switch4  : OUT std_logic;

dirA, dirB                          : OUT std_logic_vector(1 DOWNTO 0)

);

END TrainController;

ARCHITECTURE a OF TrainController IS

-- Create a new TYPE called STATE_TYPE that is only allowed

-- to have the values specified here. This

-- 1) enables using helpful names for values instead of

--    arbitrary values

-- 2) ensures that signals of this type can

-- only have valid values, and

-- 3) helps the synthesis software create

-- efficient hardware for the design.

TYPE STATE_TYPE IS (

A1CCW_B3CCW,

A1CCW_B2CCW,

A1CCW_B2CW,

Aarrived_B3CCW,

Aarrived_B2CCW,

Aarrived_B2CW

);

-- Create a signal of the new type.

SIGNAL current_state	:	STATE_TYPE;

-- This creates some new internal signals which will be

-- concatenations of some of the sensor signals.

-- This will make CASE statements easier.

SIGNAL sensor12, sensor14 : std_logic_vector(1 DOWNTO 0);

BEGIN

-- This is the sequential logic portion that is clocked

PROCESS (clock, reset)

BEGIN

IF reset = '1' THEN

current_state <= A1CCW_B3CCW;

ELSIF clock'EVENT and clock='1' THEN

CASE current_state IS

WHEN A1CCW_B3CCW =>

CASE sensor12 IS

WHEN "11" => current_state <= Aarrived_B2CCW;

WHEN "10" => current_state <= Aarrived_B3CCW;

WHEN "01" => current_state <= A1CCW_B2CCW;

WHEN others => current_state <= A1CCW_B3CCW;

END CASE;

WHEN Aarrived_B3CCW =>

CASE sensor12 IS

WHEN "11" => current_state <= Aarrived_B2CCW;

WHEN others => current_state <= Aarrived_B3CCW;

END CASE;

WHEN A1CCW_B2CCW =>

CASE sensor14 IS

WHEN "11" => current_state <= Aarrived_B2CW;

WHEN "10" => current_state <= Aarrived_B2CCW;

WHEN "01" => current_state <= A1CCW_B2CW;

WHEN others => current_state <= A1CCW_B2CCW;

END CASE;

WHEN Aarrived_B2CCW =>

CASE sensor4 is

WHEN '1' => current_state <= Aarrived_B2CW;

WHEN others => current_state <= Aarrived_B2CCW;

END CASE;

WHEN A1CCW_B2CW =>

CASE sensor12 is

WHEN "10" => current_state <= Aarrived_B2CW;

WHEN "01" => current_state <= A1CCW_B2CCW;

WHEN others => current_state <= A1CCW_B2CW;

END CASE;

WHEN Aarrived_B2CW =>

current_state <= Aarrived_B2CW;

END CASE;

END IF;

END PROCESS;

-- The following logic is NOT in a process block,

-- and thus does not depend on any clock.

-- Everything here is pure combinational

-- logic, and exists in parallel with everything else.

-- These outputs are constant values for this solution;

Switch3 <= '0';

Switch4 <= '0';

-- Combine bits for the internal signals declared above.

-- ("&" operator is concatenation)

sensor12 <= sensor1 & sensor2;

sensor14 <= sensor1 & sensor4;

--since this is moore FSM,

--the outputs only depend on the current state

WITH current_state SELECT switch1 <=

'1' WHEN A1CCW_B2CCW,

'1' WHEN Aarrived_B2CCW,

'0' WHEN Aarrived_B2CW,

'1' WHEN A1CCW_B2CW,

'0' WHEN others;

WITH current_state SELECT switch2 <=

'0' WHEN A1CCW_B2CCW,

'0' WHEN Aarrived_B2CCW,

'0' WHEN Aarrived_B2CW,

'0' WHEN A1CCW_B2CW,

'0' WHEN others;

WITH current_state SELECT dirA <=

"01" WHEN A1CCW_B3CCW,

"00" WHEN Aarrived_B3CCW,

"01" WHEN A1CCW_B2CCW,

"00" WHEN Aarrived_B2CCW,

"00" WHEN Aarrived_B2CW,

"01" WHEN A1CCW_B2CW;

WITH current_state SELECT dirB <=

"01" WHEN A1CCW_B3CCW,

"01" WHEN Aarrived_B3CCW,

"01" WHEN A1CCW_B2CCW,

"01" WHEN Aarrived_B2CCW,

"10" WHEN Aarrived_B2CW,

"10" WHEN A1CCW_B2CW;

END a;